DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ENCLOSURE NO. 1 TO CONTRACT NO. 25-149

I.  Cupping System Rated 350 S.P.M. and typical operating speed of 285-300 S.P.M pending Die progression

ITEM I:  CUPPING SYSTEM EQUIPMENT DESCRIPTION

A. One (1) Perfecto swing away, high-speed servo feed assembly

B. One (1) Feed mounted Apex Unist Lubricator

C. One (1) new DACH 165 Minster high-speed, double-action, Dynamically Balanced Hydro Statically Guided Cupping Press with hydraulic brake/clutch assembly.

D. One (1) floor Mounted Hydraulic Press lubrication and clutch system

E. One (1) set of four (4) press vibro pads

F. Stainless Steel air conveyor blower assembly

G. One (1) electrical control package. Complete electronic control package with Allen Bradley Control Logix. The control console and motor starter cabinet are included with this package.

H. Strong Arm Electrical Control Console System

I. Rittal or Hoffman Equivalent MCC enclosure with AC unit

J. Power Monitor System

K. One (1) lot guards for the press front, back and sides. Electrical interlock is provided to prevent the operation of press when protection devices are not in place.

L. One (1) lot press wiring. This item covers the labor and material for wiring base press junction boxes as received from the press manufacturer in order to provide the operational functions. Includes remote operation functions, top stop and E-stop push buttons on both the front and back of press.

M. One (1) lot press pneumatic plumbing. Labor and material required to completely plumb the base press in order to provide the necessary pneumatic functions required for the die design.

N. One (1) sets of die roll in rails

O. One (1) swing-away scrap hood for die

P. One (1) lot rigging, assembly and labor to include set up of system at our Canton site and operate at commercial speeds for customer acceptance.

Q. One (1) lot system documentation to include, buy off acceptance settings, equipment and die manuals, assembly drawings and bills of materials.

R. One (1) Magnetic Quad Clamping for inner slide with controls and safeties for quick die removal.

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 1 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

The fixture must be fixed to the floor while it is being used for safety and a safe floor area must be marked around the fixture. It is supplied with predrilled holes and the floor fixing to attach it.

| [IMAGE placeholder] | [IMAGE placeholder] | [IMAGE placeholder] | [IMAGE placeholder] |
| --- | --- | --- | --- |

The project cost is based on existing engineering. If the scope of the project is altered by the customer, Stolle reserves the right to adjust the cost and/or delivery schedule to accommodate the changes. If the customer is to incur additional expense for the project, Stolle will submit an estimate for approval prior to proceeding. Note, shipping charges to be prepaid and invoiced. Work will not begin work on this project until a correct (hard copy) purchase order payment is received.

D1: Critical Spares-15 Out (269) FIT

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

E1: Critical Spares 15-out 33cl

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

F1: Critical Spares 14-out (355cl)

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

[IMAGE placeholder: Signature Stamp]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 6 |
| --- | --- |