DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ITEM II.

S. One (1) Stolle Specially Designed Sixteen (16) out (269ml) Fit Cup Die Set complete with Stolle supplied carbide tooling for production of cups. Die set to incorporate Patented Progressive Piston (non-sequential forming or blanking) for reduction of press loads and vibrations and increased tool life.

Die set to include the following:

- Pneumatic stock plate assemblies with anti-rotation piston retainer assemblies

- Gun drilled die set for removal of die holder stock plate air fittings

- Hard chrome plated stock plates

- Internal scrap cutter assembly

- Stolle’s patented Progressive Piston and Cylinder arrangement with S-7 shock steel components

- One (1) piece outer guide pin assemblies with automatic lubrication features

- One (1) piece inner guide pin assemblies with automatic lubrication features

- Nylon entrance rails with stand-alone stock guides

- Complete set of carbide wear tools including cut edges, blank and draw pad.

- Hydraulic assist package on inner punch for installation of die

- Inner draw punch adapters

- Registered, fixed blank and draw die retainers aligned to outer die shoe concentric to within .0005”

- Die set designed and machined for .020” web F/B – L/R

- Cup markers for marking the sheet prior to the blank and draw operation

- Die set machined for non-round cut edges and blank and draw dies

- Necessary mounting bolts and fasteners

- Indicating fixture

T. One (1) lot Sixteen (16) out air cup conveyor package with bottom mount sensors

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 2 |
| --- | --- |