DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ENCLOSURE NO. 1 TO CONTRACT NO. 25-149

I.  Cupping System Rated 350 S.P.M. and typical operating speed of 285-300 S.P.M pending Die progression

ITEM I: CUPPING SYSTEM EQUIPMENT DESCRIPTION

A. One (1) Perfecto swing away, high-speed servo feed assembly

B. One (1) Feed mounted Apex Unist Lubricator

C. One (1) new DACH 165 Minster high-speed, double-action, Dynamically Balanced Hydro Statically Guided Cupping Press with hydraulic brake/clutch assembly.

D. One (1) floor Mounted Hydraulic Press lubrication and clutch system

E. One (1) set of four (4) press vibro pads

F. Stainless Steel air conveyor blower assembly

G. One (1) electrical control package. Complete electronic control package with Allen Bradley Control Logix. The control console and motor starter cabinet are included with this package.

H. Strong Arm Electrical Control Console System

I.  Rittal or Hoffman Equivalent MCC enclosure with AC unit

J.  Power Monitor System

K. One (1) lot guards for the press front, back and sides. Electrical interlock is provided to prevent the operation of press when protection devices are not in place.

L. One (1) lot press wiring. This item covers the labor and material for wiring base press junction boxes as received from the press manufacturer in order to provide the operational functions. Includes remote operation functions, top stop and E-stop push buttons on both the front and back of press.

M. One (1) lot press pneumatic plumbing. Labor and material required to completely plumb the base press in order to provide the necessary pneumatic functions required for the die design.

N. One (1) sets of die roll in rails

O. One (1) swing-away scrap hood for die

P. One (1) lot rigging, assembly and labor to include set up of system at our Canton site and operate at commercial speeds for customer acceptance.

Q. One (1) lot system documentation to include, buy off acceptance settings, equipment and die manuals, assembly drawings and bills of materials.

R. One (1) Magnetic Quad Clamping for inner slide with controls and safeties for quick die removal.

[IMAGE placeholder: DS Signature Box]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 1 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ITEM II.

S. One (1) Stolle Specially Designed Sixteen (16) out (269ml) Fit Cup Die Set complete with Stolle supplied carbide tooling for production of cups. Die set to incorporate Patented Progressive Piston (non-sequential forming or blanking) for reduction of press loads and vibrations and increased tool life.

Die set to include the following:

- Pneumatic stock plate assemblies with anti-rotation piston retainer assemblies

- Gun drilled die set for removal of die holder stock plate air fittings

- Hard chrome plated stock plates

- Internal scrap cutter assembly

- Stolle's patented Progressive Piston and Cylinder arrangement with S-7 shock steel components

- One (1) piece outer guide pin assemblies with automatic lubrication features

- One (1) piece inner guide pin assemblies with automatic lubrication features

- Nylon entrance rails with stand-alone stock guides

- Complete set of carbide wear tools including cut edges, blank and draw pad.

- Hydraulic assist package on inner punch for installation of die

- Inner draw punch adapters

- Registered, fixed blank and draw die retainers aligned to outer die shoe concentric to within .0005”

- Die set designed and machined for .020” web F/B – L/R

- Cup markers for marking the sheet prior to the blank and draw operation

- Die set machined for non-round cut edges and blank and draw dies

- Necessary mounting bolts and fasteners

- Indicating fixture

T.  One (1) lot Sixteen (16) out air cup conveyor package with bottom mount sensors

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 2 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ITEM III:

U. One (1) Stolle Specially Designed Fifteen (15) out (33cl) Standard Cup Die Set complete with Stolle supplied carbide tooling for production of cups. Die set to incorporate Patented Progressive Piston (non-sequential forming or blanking) for reduction of press loads and vibrations and increased tool life.

Die set to include the following:

- Pneumatic stock plate assemblies with anti-rotation piston retainer assembliesPneumatic stock plate assemblies with anti-rotation piston retainer assemblies

- Gun drilled die set for removal of die holder stock plate air fittings

- Hard chrome plated stock plates

- Internal scrap cutter assembly

- Stolle’s patented Progressive Piston and Cylinder arrangement with S-7 shock steel components

- One (1) piece outer guide pin assemblies with automatic lubrication features

- One (1) piece inner guide pin assemblies with automatic lubrication features

- Nylon entrance rails with stand-alone stock guides

- Complete set of carbide wear tools including cut edges, blank and draw pad.

- Hydraulic assist package on inner punch for installation of die

- Inner draw punch adapters

- Registered, fixed blank and draw die retainers aligned to outer die shoe concentric to within .0005”

- Die set designed and machined for .020” web F/B – L/R

- Cup markers for marking the sheet prior to the blank and draw operation

- Die set machined for non-round cut edges and blank and draw dies

- Necessary mounting bolts and fasteners

- Indicating fixture

V.  One (1) lot Fifteen (15) out air cup conveyor package with bottom mount sensors

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 3 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ITEM IV:

W. One (1) Stolle Specially Designed Fourteen (14) out (355ml) Cup Die Set complete with Stolle supplied carbide tooling for production of cups. Die set to incorporate Patented Progressive Piston (non-sequential forming or blanking) for reduction of press loads and vibrations and increased tool life.

Die set to include the following:

- Pneumatic stock plate assemblies with anti-rotation piston retainer assemblies

- Gun drilled die set for removal of die holder stock plate air fittings

- Hard chrome plated stock plates

- Internal scrap cutter assembly

- Stolle’s patented Progressive Piston and Cylinder arrangement with S-7 shock steel components

- One (1) piece outer guide pin assemblies with automatic lubrication features

- One (1) piece inner guide pin assemblies with automatic lubrication features

- Nylon entrance rails with stand-alone stock guides

- Complete set of carbide wear tools including cut edges, blank and draw pad.

- Hydraulic assist package on inner punch for installation of die

- Inner draw punch adapters

- Registered, fixed blank and draw die retainers aligned to outer die shoe concentric to within .0005”

- Die set designed and machined for .020” web F/B – L/R

- Cup markers for marking the sheet prior to the blank and draw operation

- Die set machined for non-round cut edges and blank and draw dies

- Necessary mounting bolts and fasteners

- Indicating fixture

X.  One (1) lot Fourteen (14) out air cup conveyor package with bottom mount sensors

ITEM V:

Y. One (1) Stolle Specially Designed Thirteen (13) out (473ml) Standard Cup Die Set complete with Stolle supplied carbide tooling for production of cups. Die set to incorporate Patented Progressive Piston (non-sequential forming or blanking) for reduction of press loads and vibrations and increased tool life.

Die set to include the following:

- Pneumatic stock plate assemblies with anti-rotation piston retainer assemblies

- Gun drilled die set for removal of die holder stock plate air fittings

- Hard chrome plated stock plates

- Internal scrap cutter assembly

- Stolle’s patented Progressive Piston and Cylinder arrangement with S-7 shock steel components

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 4 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

- One (1) piece outer guide pin assemblies with automatic lubrication features

- One (1) piece inner guide pin assemblies with automatic lubrication features

- Nylon entrance rails with stand-alone stock guides

- Complete set of carbide wear tools including cut edges, blank and draw pad.

- Hydraulic assist package on inner punch for installation of die

- Inner draw punch adapters

- Registered, fixed blank and draw die retainers aligned to outer die shoe concentric to within .0005”

- Die set designed and machined for .020” web F/B – L/R

- Cup markers for marking the sheet prior to the blank and draw operation

- Die set machined for non-round cut edges and blank and draw dies

- Necessary mounting bolts and fasteners

- Indicating fixture

Z. One (1) lot Thirteen (13) out air cup conveyor package with bottom mount sensors

ITEM VI: COIL HANDLING EQUIPMENT (Set up Initial Configuration)

A1. Coil Handling Equipment

- Perfecto Model UTH308074 Traversing Upender

- Perfecto Model LC30NR Coil Car

- (1) Perfecto Model RD308074M Double End Uncoiler

- (1) Perfecto Model MS74 Material Stand

- Interconnect Cables

- weight Scale

- CE Certification

Note: Weight Scale does not go trough PLC. It has its own readout and will show up on HMI

C1. Die Set Safety Turnover Unit

The fixture is designed to turn over all 3 plates of the die set individually. For loading the plates into the DIE plate cradle adaptor bars are used. The frame is lifted and placed over the die plate with a forklift truck, to allow accessibility if required the lifting pockets have been made detachable.

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 5 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

The fixture must be fixed to the floor while it is being used for safety and a safe floor area must be marked around the fixture. It is supplied with predrilled holes and the floor fixing to attach it.

[IMAGE placeholder]

The project cost is based on existing engineering. If the scope of the project is altered by the customer, Stolle reserves the right to adjust the cost and/or delivery schedule to accommodate the changes. If the customer is to incur additional expense for the project, Stolle will submit an estimate for approval prior to proceeding. Note, shipping charges to be prepaid and invoiced. Work will not begin work on this project until a correct (hard copy) purchase order payment is received.

D1: Critical Spares-15 Out (269) FIT

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

E1: Critical Spares 15-out 33cl

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

F1: Critical Spares 14-out (355cl)

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

DS

CC

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 6 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

G1: Critical Spares 13-out (473cl)

- (3) Cut-Edge

- (3) Blank & Draw Die

- (3) Draw Pad

- (3) Die Center

- Set of Seals

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 7 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

II. Nine (9) STANDUN BOBYMAKERS

[IMAGE placeholder]

DS CC

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 8 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

SECTION 1: STANDARD BODYMAKER DESCRIPTION

The Standun is the only Bodymaker designed for easy conversion to produce the most popular beverage can sizes up to 500ml. Production speeds determined by the tool pack length, can diameter, Bodymaker mechanical configuration, as well as plant process controls.

| Description | B6-B7 |
| --- | --- |
| Stroke | 24 inches |
| Width | 6 feet |
| Height | 7 feet |
| Length | 13 feet, 5 inches |
| Weight | 19,500 pounds |
| Foundation | Floor Level |
| Speed | Up to 300-400 strokes per minute (See Note 1 below) |
| Toolpack Length | TBD |
| Can Diameter | 211 and 204 |
| Max. Untrimmed Can Height (aluminum) | TBD |

Standard Mechanical Features of a Standun Bodymaker include:

Equipment runs a single diameter length can for aluminum with one set of lifting swivel hoists, can discharge conveyor, foundation mounting hardware, and built to CE safety standards with English language signs/literature.

*Note 1 – The Standun Bodymaker speed is rated in Strokes Per Minute (SPM). This is the Engineered MAX Speed the Mechanical Systems in the Machine are rated to. This Rating is WITHOUT the process inputs or material in the plant. And those items will ultimately determine the OPERATIONAL Speed of the machine. And it is the plants resposibility to determine the most efficent Speed during Operation in the plant.

Includes:

- Mist Collection Point

- Flanged Ram

- Pride Domer Assembly

- Pneumatic Clutch Brake (Goizper)

- Poly Insert Type Stripper

- Pride Modular Tool Pack

- On-Machine Hydraulic Package

- Carbide Face Fixed Redraw Sleeve

Customer should select one (1) option for each category below:

| Machine Hydraulics: | Ram: | Trimmer Type: |
| --- | --- | --- |
| On-Machine Hydraulics to be provided unless noted. | Flange Ram is provided for 2.5” and 2” can diameters unless noted. | ☐ Stolle ☐ Belvac ☐ CMB |

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 9 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

Standard Electrical Features of a Standun Bodymaker include: Allen Bradley Safety PLC, high-speed logic functionality integrated in PLC program, interconnect cables between panel & machine (100 ft.), capable of operating in 50-degree C ambient temperature with NEMA frame motors, Brady heat shrink wire markers, & power wires will be sized according to load.

Includes:

- Allen Bradley Control Logix L8ES & L8SP PLC

- Studio5000 Version 31 program, and FactoryTalkView Machine Edition Version 8.2

- (2) 1756-EN2T Ethernet modules: (1) Line Control/Data Collection & HMI, (1) VFD’s and Point I/O

- Integrated Programmable Limit Switch functionality using AMCI 8213 Resolver Interface module

- AMCI HT-400 Resolver

- Allen Bradley 1734 series Point Guard I/O modules using CIP Safety protocol over Ethernet/IP (mounted on machine and operator console)

- Main Circuit Breaker: Allen Bradley 140 Series w/ flange mounted cable operator

- 24V DC Power Supply: Allen Bradley 1606 series w/ 3 phase input

- Door Mounted Network Interface: Grace Port with Ethernet & 5-amp GFCI service outlet

- Allen Bradley 700S-CF & 100S-C series safety control relays & contactors

- Allen Bradley 700-CF & 100-C series non-safety control relays and contactors

- Allen Bradley 140M Series motor circuit protection

- Allen Bradley 1492 / 1489 Series circuit breakers as appropriate

- Pilz 777949 for AC Drive applications Back EMF style for Trimmer motion detection

- Electrically Timed Air Strip (MAC 56 series solenoid valve)

- Main Drive VFD: Allen Bradley PowerFlex 755 with Safe-Off feature & Ethernet/IP

- Trimmer Drive VFD: Allen Bradley PowerFlex 525with Safe-Off feature & Ethernet/IP

- Hoffman Spectracool control panel air conditioner

- Hoffman NEMA type 12 control panel

- Sencon smart prox w/ BCM 387 module short can sensor

- Euchner CET-AR solenoid latching guard switches

- Allen Bradley 194E series with black/gray lockable handle & aux contact machine mounted motor disconnects

- Allen Bradley 800TC series (30mm) pushbuttons & LED indicators

- Sourcing (PNP) 24VDC sensor wiring standard

- Inductive prox sensors: Turck

- Pressure Sensing: IFM Efector PP series – I/O Device Link

- Flow Sensing: IFM Efector SM & SI series flow meters

- Status Beacon (24V DC): Allen Bradley 856T (Red / Amber / Green)

- Terminal blocks (sized according to wire gauge and load requirements): spring cage on control (Allen Bradley 1492-L series), screw terminal for power (Allen Bradley 1492-J series)

- Floor standing operator console – Allen Bradley Panel View Plus 7 Standard 1000 HMI on Ethernet

- Motors: NEMA (inch frame) Toshiba EQP III – Main Frame 75HP / Main Pump 15HP / Recirculation Pump 2HP

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 10 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

SECTION 2: ADDITIONAL MACHINE OPTIONS

| Selected | Mechanical Options |
| --- | --- |
| ☐ | Stripper: LPT Ceramic (Lieb) |
| ☒ | Interconnecting crossover gravity trackwork with SS guard $25,345.00 each |
| ☐ | Redraw Sleeve: Ceramic Face |
| ☒ | Domer Assembly: CMB Domer Assembly $12,000.00 each |
| ☐ | Domer Spring: Yellow (Ram Control) |
| ☐ | Domer Spring: Red (Low Shock) |
| ☐ | Domer Spring: Blue (High Strength) |
| ☒ | Foundation Anchor assembly template |
| ☐ | Hydraulic Cart with locking casters |
| ☐ | Pride Guardian On Machine |
| ☐ | Machine Certifications: (i.e. CE, NR12, UL, GCC) The Standun Bodymaker Machine Certification is your guarantee of compliance with the Directives on the approximation of the laws of the Member States relating to machinery for any specific certification requested. Consists of: - Machine Safety Assessment and Performance Evaluation - EMC Performance Evaluation; Audible Performance Levels Testing - Dual Language Translations for display & PB legend plates |

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 11 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

| Selected | Electrical Options |
| --- | --- |
| ☐ | IEC (inch frame) ABB (Alternate Standard Equipment, Required for CE) |
| ☐ | Fortress interlock solenoid latching guard switches |
| ☐ | No motor controls on HMI |
| ☐ | 4 pole main circuit breaker |
| ☐ | 65 kAIC rated main panel |
| ☐ | Rittal control panel air conditioner |
| ☐ | DC Power Supplies sized for 200% of Load |
| ☐ | Rittal control panel |
| ☐ | Remove Interconnect cables between panel & machine |
| ☐ | Screw terminal for control and power (Included w/ CE certification) |
| ☐ | EU wire color standard (Included w/ CE certification) |
| ☐ | Permatex Partex Type PA wire labels |
| ☒ | Short Can Sensor: Sencon 487 programmable Smart Prox. |
| ☐ | Short Can Sensor: Steel Cans – Sencon # 11-267-03 |
| ☒ | Inch Station $1,900.00 each |
| ☐ | Non-Standard Wire Colors |
| ☒ | Utility Monitoring Package: $7,440.00 each Consists of: - Allen Bradley series 1408 Power Monitor 1000 - IFM Efector SD series compressed air flow & vacuum flow monitor for Trimmer |
| ☒ | Allen Bradley Articulated Pendant $8,000.00 each Consists of: - StrongArm articulated pendant (floor or overhead mount) - Allen Bradley Panel View Plus 700 on Ethernet |
| ☒ | Additional Language (Manual & Tags) Each (Specify on – Project Details) Equipment manuals can be provided in English, German, Spanish, French, Portuguese or Chinese for no additional cost. Manual translations in all other languages may incur additional charges. |

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 12 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

SECTION 4: TOOLS & FIXTURES

| Selected | Logistics | Price (USD) | Qty. |
| --- | --- | --- | --- |
| ☒ | Mandatory Tools & Fixtures | $19,340.00 | 1 |
| ☒ | Recommended Additional Tools & Fixtures | $20,450.00 | 1 |
| ☐ | Punch Cross Hatch Machine |  |  |
| ☐ | Pride Guardian II Portable Unit |  |  |

III. Two (2) Standard 36MRT Decorator with 8 ink stations

[IMAGE placeholder]

DS / CC

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 13 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

STOLLE MACHINERY COMPANY LLC	CANPACK COLOMBIA S.A.S.

STANDARD FEATURES

| STANDARD MECHANICAL FEATURES: |  |
| --- | --- |
| 1. Can size operating speed | 2,200CPM 211X355ml |
| 2. Mandrels | 36 |
| 3. Mandrel material | Light-weight steel self-aligning |
| 4. Mandrel trip | Single mandrel (rotary trip) |
| 5. Trip arm pivot | Needle bearing with lube |
| 6. Inkers | 8 interchangeable with mechanical drive. |
| 7. Inker cooling | Inkers equipped to accept optional chillers |
| 8. Oil cooling | Ink station main frame & critical gearboxes |
| 9. Infeed | Air assist (5 HP blower) adjustable |
| 10. Can stop | Pneumatic |
| 11. Plate cylinder | Light weight magnetic |
| 12. Running registration | Included |
| 13. Blanket wheel | 12 segments |
| 14. Overvarnish unit | Reverse type 10 HP AC Vector Duty |
| 15. Overvarnish pre-spin | Flat Belt |
| 16. Overvarnish day tank | 8 gallon with pump |
| 17. Transfer | 36 suction heads |
| 18. Oven drive clutch | Included |
| 19. Guards | Interlocked |
| 20. |  |
| STANDARD ELECTRICAL FEATURES: |  |
| 1. Main Motor | 100 HP AC TEBC – Inverter duty with mechanical drive inkers. |
| 2. Drive | Allen Bradley power flex 755 |
| 3. Logic | Control Logix |
| 4. Timing Device | PLS – integrated in PLC |
| 5. Tropicalized motors | Included |
| 6. Interconnecting cables | Included |
| ELECTRICS: |  |
| 460V/3PH/60Hz or as specified by Buyer |  |

[IMAGE placeholder]

ENCLOSURE NO. 1 TO CONTRACT NO. 25-149	PAGE NO. 14

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

STOLLE MACHINERY COMPANY LLC	CANPACK COLOMBIA S.A.S.

## Change partsChange parts

| Change Parts Size | 211 diameter |  | 211x355 |
| --- | --- | --- | --- |
|  | Price | Qty | Extended |
| Adjustable infeed | $17,000 | 2 | $34,000 |
| Mandrels light wt. self-align | $820 | 76 | $62,320 |
| Quick change pockets set 36 mandrel | $9,900 | 2 | $19,800 |
| Applicator roll tire 1 ea. | $2,300 | 2 | $4,600 |
| Gravure roll | $5,400 | 0 | $0 |
| Fountain | $8,500 | 0 | $0 |
| Tooling | $2,100 | 2 | $4,200 |
| Total |  |  | $124,920 |
| Change Parts Size | Addational 211 size |  | 211x33 |
|  | Price | Qty | Extended |
| Mandrels light wt. self-align | $820 | 76 | $62,320 |
| Quick change pockets set of 36 | $9,900 | 2 | $19,800 |
| Applicator roll tire 1 ea. | $2,300 | 2 | $4,600 |
| Total |  |  | $86,720 |
| Change Parts Size | sleek and slim |  | 211x473 |
|  | Price | Qty | Extended |
| Mandrels light wt. self-align | $820 | 76 | $62,320 |
| Quick change pockets set of 36 | $9,900 | 2 | $19,800 |
| Applicator roll tire 1 ea. | $2,300 | 2 | $4,600 |
| Total |  |  | $86,720 |
| Change Parts Size | Lengh sleek and slim |  | 204x269 (202) |
|  | Price | Qty | Extended |
| Mandrels light wt. self-align | $870 | 76 | $66,120 |
| Quick change pockets set of 36. | $9,900 | 0 | $0 |
| Applicator roll tire 1 ea. | $2,300 | 0 | $0 |
| Total |  |  | $66,120 |

IV. DG 250 Inside Spray Machines

[IMAGE placeholder]

ENCLOSURE NO. 1 TO CONTRACT NO. 25-149	PAGE NO. 15

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

[IMAGE placeholder]

DG 250 Inside Spray Machines

- DG 250 Inside Spray Machines - Ten single machines

- Starwheels- two per machine- UHMW solid starwheel

- Guns- two per machine - Nordson MEG II

- Lacquer Pressure Control - Nordson Spray Pressure Control w/ Dual Filters for Ten machines

- Rittal 5 Bay Control Pnl with A/C - Included

- iTrax - Nordson Itrax SC and SM modules for Fourteen (14) machines

- iTrax Discharge Can Stops - Included for Ten (10) machines

- iTrax PRx (Servo Pressure Regs & Transducers) - Included for Ten (10) Machines

- Control Panel - Single Panel for Fourteen (14) machines

- Cards – Two (2) Ethernet Cards

- Pumps – Two (2) Dual Nordson EP pump w Stands & VFDs

- Nordson Inkdot on machines – not included

- Nordson Inkdot Controls – not included

- Spray Material Day Tank - Customer Supplied

- Exhaust System Transition - Included for Ten (10) machines

- Piping, wires, cables and wire ways - Customer Supplied

- Spinner Motor VFD’s - Included for Fourteen (14) machines

- Spray Gun Nozzles - Included (Customer to supply part numbers)

- Heater/TCU - Dual Zone TCU Included

- Utility Monitoring (Power, Air & Vac) - Included

- 204 Change Parts (26.9cl) - Included for Ten machines

- Set-Up Gauges - Included for 204 and 211 Can Diameters

- 65 KAIC Main Disconnect - Included

- Can count logic - Included

- Tropicalized Motors - Included

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 16 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

| Description |  |
| --- | --- |
| INSIDE SPRAY SYSTEMS | Individual Tall Cabinet ISMs |
| DG 250 inside sprays | Dual gun |
| Speed | Estimated machine speed is 350CPM on 211 Can Diameter, max Can speed is Spray Time and Can Size dependent |
| Piping, air, vacuum and spray | Customer Supplied |
| Pumps | EP Pumps Included |
| Can Height (max) | 710 |
| Can Diameter | 211 |
| Platform | Customer Supplied |

FEATURES

| MECHANICAL FEATURES: |  |
| --- | --- |
| Guards | Stainless Steel Glass enclosed, and interlocked |
| Filter | Dual filter supplied |
| Gearbox | Camco index |
| Pumps | Nordson EP Pumps |
| Starwheels | Solid, UHMW |
| Gun Mounts | Swing away |
| Exhaust | Transition Only |
| Guns | Nordson MEGII Guns (2) |
| Platform | Customer Supplied |
| Heater/ Cooler | Dual Zone TCU |
| Vacuum filter | Included |
| Can stop | Included |
| Vacuum saver valve | Included |
| Operator manual | One standard manual- Electronic Copy |
| Nozzles | Included (Customer to supply part numbers) |
| ELECTRICAL FEATURES: |  |
| Panel | Hoffman panel sized for quoted number of machines |
| - Controls | Allen Bradley Control, Nordson gun drivers |
| - Timers | PLC |
| - Relays | Allen Bradley 700 series |
| - Drives | Allen Bradley |
| Controls | Allen Bradley Control, Nordson gun drivers |
| Operator panel (HMI) | Pushbutton station at each machine |
| Motors | 1 HP main drive, 1/3 HP spinner drive |
| Gun drivers | Nordson |
| ELECTRICS: |  |
| Line | As specified |
| Control | 24 VDC |

[IMAGE placeholder]

| ENCLOSURE NO. 1 TO CONTRACT NO. 25-149 | PAGE NO. 17 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ENCLOSURE NO. 2 TO CONTRACT NO. 25-149

- The Seller shall provide to the Buyer the following documentation for the Equipment:

  - The General Arrangement Drawings shall show general arrangements of the equipment, and piping, wiring and method of assembly. These drawings will be returned to the Seller marked „Accepted” or „Revision Required”. In case of revision of the drawings by the Buyer is required, the Seller shall agree the drawings with the Buyer and obtain its acceptance. Acceptance of the drawings will, in no way, relieve the Seller from the responsibility of supplying properly designed Equipment in compliance with the specification indicated in the Contract and Enclosures thereto on the delivery date indicated in the Contract. In the event of acceptance the drawing by the Buyer, no changes shall be made to the general arrangement of the Equipment covered by the Contract without written notification to the Buyer.

  - After acceptance by the Buyer the Seller shall provide to the Buyer two (2) sets of the Documentation defined in point 1.1.

  - Validated Drawings: Upon receipt of General Arrangement marked „Accepted” by the Buyer, the Seller shall submit two (2) sets of validated electrical and mechanical drawings to the Buyer. The following are to be received by the Buyer no later than 12 weeks after signature of this Contract.

- Panel Wiring Schematics

- Equipment Wiring Schematics

- Control Panels, Dimensions and Locations

- Power Panels, Dimensions and Locations

- Junction Boxes, Dimensions and Locations

- Miscellaneous Electrical Panels, Dimensions and Locations

- All Additional Miscellaneous Electrical Drawings Not Previously Addressed

- Bolt hole Patterns, Size and Dimensions Layout and Gross Weight of Equipment Within Ten (10) Percent

- Can Conveyance Interface Locations (Outfeed and Infeed), as Applicable

- Utility Connections (Pipe Size and Well Defined Locations), For Vacuum and compressed Air, Water, Hot Water, Caustic/Chemical, Lubes, Electrical, etc., as required.

- Installation Drawing and Equipment Assembly Technical information

If there are revisions requested by the Buyer of validated drawings, the Seller shall supply updated drawing within 4 weeks from „Revision Requested”.

  - Final As Built Drawings:

    - Upon delivery of equipment as agreed upon in the Contract, the Seller shall supply the Buyer with one (1) set of all electrical and mechanical drawings and the Equipment PLC program to represent the Equipment „as built”, four weeks after Equipment ship date.

  - Operation and Maintenance Manual Requirements:

The Seller shall provide to the Buyer three (3) sets of operation and maintenance manuals for each type of Equipment ordered in Spanish and English Language, to be received no later than six (6) weeks after buy – off of the Equipment:

The above shall include, but not be limited to, the following:

- Description: A detailed description of the Equipment function and sequence of operation of Equipment.

[IMAGE placeholder]

| ENCLOSURE NO. 2 TO CONTRACT NO. 25-149 | PAGE NO. 1 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

- Adjustments: A complete description of each adjustment, including the timing of each function, the proper setting for all utilities, flow rates, proper tooling settings and electrical timing diagrams.

- Troubleshooting:  A complete listing of operational problems, their cause and the remedy, as well as the resulting effect on the finished product.

- Operation and Maintenance: A complete comprehensive set of start up, operating and shut down instructions shall be included. This section shall also cover required schedules of routine maintenance lubrication. A schematic of the lubrication system shall be provided indicating all points of lubrication. A recommended type and supplier of lubricating materials shall be included. Recommendations for preventative maintenance frequency and special tasks shall also be included.

1.6.	A list of recommended spare parts for one (1) year of operation shall be provided to the Buyer ten weeks (10) from the receipt by the Seller from the Buyer, the Buyer’s product drawing (can/cups) Drawings with specification.

[IMAGE placeholder]

| ENCLOSURE NO. 2 TO CONTRACT NO. 25-149 | PAGE NO. 2 |
| --- | --- |

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

Enclosure no 3 to the contract 25-149

| SMQ-07315.1 | One (1) Dynaform 165 Cupping System with Two(2) sets of Fifteen (15) out (269ml) Fit Cup Die Set, Fifteen (15) out (33cl) Standard Cup Die Set, Fourteen (14) out (355ml) Cup Die Set, Thirteen (13) out (473ml) Standard Cup Die Set and complete coil handling system with Die set safety turnover unit and Critical Spares | 1 | 5 885 310,00 | 5 885 310,00 |
| --- | --- | --- | --- | --- |
| SMQ-07316.0 | Two (2) 36MRT Concord Decorators with 8 ink stations set up for primary can 204x269ml with selected optional items and change parts 204/202x269, 211x33, 211x355, 211x473 | 2 | 2 802 080,00 | 5 604 160,00 |
| SMQ-07358.0 | Ten (10) individual DG250 Tall Cabinet Spray with Controls for twelve machines to run 211x33cl, 35,5cl & 47,3cl Can Diameters | 1 | 1 963 735,00 | 1 963 735,00 |
| SMQ-07321.0 | Nine (9) B626 Bodymakers for primary can size 211x33cl and additional can size options 211x35,5cl, 211x47,3cl, 204x26,9cl | 9 | 660 382,56 | 5 943 443,00 |
|  |  |  |  | 19 396 648,00 |

[IMAGE placeholder]

DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7

| STOLLE MACHINERY COMPANY LLC | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ENCLOSURE NO. 4 TO CONTRACT NO. 25-149

| Stolle Machinery Co, LLC | [IMAGE placeholder] |
| --- | --- |

NOTICE

Dear Stolle Machinery Customers:

When making payments to Stolle Machinery Company, LLC, Please use the following information:

| SEND CHECKS ONLY TO: Stolle Machinery Company, LLC 7329 Solution Center Chicago, IL 60677-7003 | FOR REMITS BY NEXTDAY COURIER: Stolle Machinery Co, LLC Lockbox Number 777329 350 East Devon Avenue Itasca, IL 60143 |
| --- | --- |

...................................................................................................................................................................

SEND WIRE TRANSFERS TO:

| BANK ADDRESS: | SUMITOMO MITSUI BANKING CORP. LOS ANGELES 601 S. Figueroa Street, Suite 1800 Los Angeles, CA 90017 U.S.A |
| --- | --- |
| ACCOUNT NO. | 4531689170 |
| ABA ROUTE NO. | 122041594 (Wires) |
| ABA ROUTE NO. | 122041594 (ACH) |
| *** When using an ACH payment method, please provide details of payment to your account representative) |  |
| BENEFIT: | Stolle Machinery Company, LLC |
| S.W.I.F.T | SMBCUS33 |

PLEASE REVIEW ALL YOUR RECORDS AND CORRECT.

[IMAGE placeholder]

Rodrigo Pizani

Corporate Executive V.P. Finance

Stolle Machinery Company

| ENCLOSURE NO. 4 TO CONTRACT NO. 25-149 | PAGE NO. 1 | DS CC |
| --- | --- | --- |

| DocuSign Envelope ID: D1328ABD-166C-40C3-9485-A06A729A7BB7 | CANPACK COLOMBIA S.A.S. |
| --- | --- |

ENCLOSURE NO. 5 TO CONTRACT No. 25-149

DELIVERY ACCEPTANCE PROTOCOL

in accordance with CONTRACT NO. 25-149

date..............

| Buyer: CANPACK Colombia S.A.S. Tocancipa (Cundinamarca), Km 1.5 Via Briceno Zipaquira Vereda Verganzo S. Tibitoc, Colombia | Seller: STOLLE MACHINERY COMPANY LLC 6949 South Potomac St. Centennial, CO 80112-4036, USA |
| --- | --- |

CANPACK Colombia S.A.S and STOLLE MACHINERY COMPANY LLC, hereinafter referred to as the “Parties”,

hereby confirm that the following pieces of the Equipment indicated in par. 1.1 of the Contract:

have been delivered pursuant to par. 2.1 of the Contract, however they have not been installed, started-up or commissioned yet.

Notwithstanding the above, the Parties agree that the Buyer shall make payment specified in par. 4.1.3 of the Contract, i.e. in the amount of 10% of the price of a piece of Equipment being subject to this Delivery Acceptance Protocol, within thirty (30) days following execution of this Delivery Acceptance Protocol, even though the Final Acceptance Test has not been performed as determined in Section 12 of the Contract No. 25-149.

For the avoidance of doubt, the Parties agree that effecting by the Buyer of the above payment does not in any way relieve the Seller from its responsibility for the compliance of the Equipment with the Contract and that any and all Buyer’s rights related to the warranties granted by the Seller based on the Contract are maintained.

| For CANPACK Colombia S.A.S | For STOLLE MACHINERY COMPANY LLC |
| --- | --- |

[IMAGE placeholder]